To Focus on Your Sample:

- Press the Esc button so that the objective is as low as possible.

- Manually focus the objective up using the focus knobs on the left or right of the microscope base.  Focus away from you to move the objective up.

- Place the specimen on the stage.

- Manually raise the mechanical stage and tighten the four knobs to lock it in position.

- Use the XY stage control to position the specimen below the objective.

- Place a drop of water on the sample.

- Lower the coarse focus until the objective touches the water.

- Look through the oculars and complete the fine focus by eye.

To Remove Your Sample:

- Press the Esc button so that the objective is as high as possible.

- Manually lower the mechanical stage.

- Remove the specimen.

- Clean the 25x objective using lens paper and lens cleaner.